# 10G SFP+ Active Optical cable

# Features：

Support 10GBASE-SR/10G Fiber Channel application

- Compliant to SFP+ Electrical MSA SFF-8431

- Compliant to SFP+ Mechanical MSA SFF-8432

- Multi rate of up to 11.3Gbps

- Transmission distance up to 300m

	+3.3V single power supply

- Low power consumption

- Operating case temp Commercial: 0°C to +70 °C

- RoHS compliant

- Password protection for A0h and A2h

# Applications

- 10GBASE-SR at 10.31Gbps

- InfiniBand QDR, SDR, DDR

- Other optical links

Absolute Maximum Ratings

Table1- Absolute Maximum Ratings

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | Vcc3 | -0.5 | - | +3.6 | V |  |
| Storage Temperature | Ts | -10 | - | +70 | °C |  |
| Operating Humidity | RH | +5 | - | +85 | % | 1 |
| Receiver Damage Threshold | P Rdmg | +3.4 | - | - | dBm |  |

Note1: No condensation

Recommended Operating Conditions

Table 2- Recommended operating Conditions

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | TC | 0 | - | +70 | °C |  |
| Power Supply Voltage | Vcc | 3.14 | 3.3 | 3.47 | V |  |
| Power Supply Current | Icc | - | - | 150 | mA |  |
| Power Dissipation | Pd | - | - | 0.6 | W |  |
| Bit Rate | BR | - | 10.3125 | - | Gbps |  |
| Fiber Bend Radius | Rb | 3 | - | - | cm |  |

Electrical Characteristics

Table 3- Electrical Characteristics

| Parameter |  |  | Symbol |  | Min. | Typ. | Max. | Units |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |  |  |  |
| Differential Data Input Swing |  |  | Vin,P-P |  | 200 | - | 1600 |  | mVPP |  |
| Input Differential Impedance |  |  | ZIN |  | 90 | 100 | 110 |  | Ω |  |
| Tx_Fault | Normal Operation |  | VOL |  | 0 | - | 0.8 |  | V |  |
|  | Transmitter Fault |  | VOH |  | 2.0 | - | VCC |  | V |  |
| Tx_Disable | Normal Operation |  | VIL |  | 0 | - | 0.8 |  | V |  |
|  | Laser Disable |  | VIH |  | 2.0 | - | VCC+0.3 |  | V |  |
| Receiver |  |  |  |  |  |  |  |  |  |  |
| Differential Date Output |  |  |  | Vout | 370 | - | 1600 | mV |  |  |
| Output Differential Impedance |  |  |  | ZD | 90 | 100 | 110 | Ω |  |  |
| Rx_LOS |  | Normal Operation |  | VOL | 0 | - | 0.8 | V |  |  |
|  |  | Lose Signal |  | VoH | 2.0 | - | VCC | V |  |  |

Optical Characteristics

Table 4-Optical Characteristics

| Parameter | Symbol | Unit | Min | Typ | Max | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Optical transmitter Characteristics |  |  |  |  |  |  |
| Data Rate | DR | Gbps | 9.953 | 10.3125 | 11.3 |  |
| Center Wavelength Range | λc | nm | 820 | 850 | 880 |  |
| Laser Off Power | Poff | dBm | - | - | -45 |  |
| Launch Optical Power | P0 | dBm | -6.0 | - | - | 1 |
| Extinction Ratio | ER | dB | 3 | - | - |  |
| Spectral Width(RMS) | RMS | nm | - | - | 0.45 |  |
| Optical Receiver Characteristics |  |  |  |  |  |  |
| Data Rate | DR | Gbps | 9.953 | 10.3125 | 11.3 |  |
| Bit Error Rate | BER | dBm | - | - | E-12 | 2 |
| Overload Input Optical | PIN | dBm | 2.4 | - | - | 2 |

| Power |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Center Wavelength Range | λc | nm | 820 | - | 880 |  |
| Receiver Sensitivity in Average Power | Sen | dBm | - | - | -9.9 | 3 |
| Los Assert | LosA | dBm | -26 | - | - |  |
| Los De-Assert | LosD | dBm | - | - | -12 |  |
| Los Hysteresis | LosH | dB | 0.5 | - | - |  |

Note:

- Coupled into 50/125 MMF.

- Measured with PRBS 231-1 test pattern @10.3125Gbps.BER=10E-12

Recommended Host Board Power Supply Circuit

Figure 1, Recommended Host Board Power Supply Circuit

Recommended Interface Circuit

Figure 2, Recommended Interface Circuit

Pin arrangement

Table 5-Pin Function Definitions

Figure 3, Pin View

| Pin | Symbol | Name/Description | Notes |
| --- | --- | --- | --- |
| 1 | VEET | Module Transmitter Ground | 1 |
| 2 | TX_FAULT | Module Transmitter Fault | 2 |
| 3 | TX_DISABLE | Transmitter Disable; Turns off transmitter laser output | 3 |
| 4 | SDA | 2-Wire Serial Interface Data Line (MOD-DEF2) |  |
| 5 | SCL | 2-Wire Serial Interface Clock (MOD-DEF1) |  |
| 6 | MOD_ABS | Module Absent, connected to VEET or VEER in the module | 2 |
| 7 | RS0 | Rate Select 0, optionally controls SFP+ module receiver |  |
| 8 | RX_LOS | Receiver Loss of Signal Indication (In FC designated as Rx_LOS and in Ethernet designated as NOT Signal Detect) | 2 |
| 9 | RS1 | Rate Select 1, optionally controls SFP+ module transmitter |  |
| 10 | VEER | Module Receiver Ground | 1 |
| 11 | VEER | Module Receiver Ground | 1 |
| 12 | RD- | Receiver Inverted Data Output |  |
| 13 | RD+ | Receiver Non-Inverted Data Output |  |
| 14 | VEER | Module Receiver Ground | 1 |
| 15 | VCCR | Module Receiver 3.3 V Supply |  |
| 16 | VCCT | Module Transmitter 3.3 V Supply |  |
| 17 | VEET | Module Transmitter Ground | 1 |
| 18 | TD+ | Transmitter Non-Inverted Data Input |  |
| 19 | TD- | Transmitter Inverted Data Input |  |
| 20 | VEET | Module Transmitter Ground | 1 |

Note:

- The module ground pins are isolated from the module case.

- The pins shall be pulled up with 4.7K-10Kohms to a voltage between 3.14V and 3.46V on host board.

- The pin is pulled up to VCCT with a 4.7K-10KΩ resistor in the module.

Monitoring Specification

Figure 4, Memory Map

Mechanical Design Diagram

Unit:mm

| Cable Length L（Unit: m） | Tolerant（Unit: cm） |
| --- | --- |
| ≤1.0 | +5/-0 |
| 1.0＜L≤4.5 | +15/-0 |
| 4.5＜L≤14.5 | +30/-0 |
| ＞14.5 | +2%/-0 |

Table 6- Cable Length

Warnings

Handling Precautions: This device is susceptible to damage as a result of electrostatic discharge (ESD).

A static free environment is highly recommended. Follow guidelines according to proper ESD procedures.

Laser Safety: Radiation emitted by laser devices can be dangerous to human eyes. Avoid eye exposure to direct or indirect radiation.